# Stepper motor tuning

- Motor 17HS08-1004S

## Nema 17 Bipolar 1.8deg 13Ncm (18.4oz.in) 1A 3.5V 42x42x20mm

| V/Hz | mH | Ohm |
| --- | --- | --- |
| .033 | 3 | 3.5 |

1 amp CONFIG value 12032 dec, gatecfg1 = 0x00EC, gatecfg2 = 0xE1 (14680300 dec combined)

- Motor 8HS11-0204S

## Nema 8 Bipolar 1.8deg 1.6Ncm (2.3oz.in) 0.2A 4.8V 20x20x28mm

| V/Hz | mH | Ohm |
| --- | --- | --- |
| .0118 | 30???guess | 24 |

.6 amp CONFIG value 12032 dec, gatecfg1 = 0x00EC, gatecfg2 = 0xE1 (14680300 dec combined)

Note: on this motor changing the current (KVAL_RUN) from 80 to 200, tuning was still good

- LIN 4418S-51D-02RO

| Vs | Holding Current | Run Current | Step/s | holding torque ( oz-in) |  |
| --- | --- | --- | --- | --- | --- |
| 24 | .5 | 1 | 500 | 61 |  |

| V/Hz | mH | Ohm |
| --- | --- | --- |
| .042 | 6.55 | 4.25 |

1 amp large fet CONFIG value 0x1F00 (7936 dec) gatecfg1 = 0x00EC, gatecfg2 = 0xE0 (14680300)

762 Steps/sec

2.14 amp large fet CONFIG value 0x1F00 (7936 dec) gatecfg1 = 0x00EC, gatecfg2 = 0xE0 (14680300)