# Release Notes: B-Scope controller

### Revision 1.2

### 6/*/2013

## Revision History

| Revision Number | Revision Date | Summary of Changes | Author |
| --- | --- | --- | --- |
| 1.0 | 11/20/12 | Initial release | ELieser |
| 1.01 | 1/31/13 | Bug fixes and memory capability | ELieser |
| 1.1 | 4/24/13 | Bug fixes, user firmware update unlimited mark/find and save virtual axis | ELieser |
| 1.2 | 6/*/13 | Zero to index and additional USB communication commands | ELieser |

## Scope

| Software Component | Latest Revision | Last Updated | Summary |
| --- | --- | --- | --- |
| Controller Firmware | 1.2 | 6/*/13 | Changed zeroing routine to zero to index. Added flipper mirror USB control and query Added limit response when limit hit. **Note: This rev will break zeroing functionality on Z-obj axis on all scopes shipped prior to 7/1/13** |
| Joystick Firmware | 1.1 | 4/24/13 | No changes |
| Windows Utility | 1.2 | 6/*/13 | Added Flipper mirror control and readout and limit indicators. |
| Windows Driver | 1.0 | 11/7/12 | No changes |

## Bugs

| Bug | Status |
| --- | --- |
| Zeroing command gets stuck after hitting second limit. | New zeroing routine implemented. |
| If one axis is on a limit sometimes other axes think they on limits as well. Intermittent on two scopes that I’ve seen. | Modifications were made to how limits are handled. Testing shows this problem is fixed. |

## New Features

| Feature | Details |
| --- | --- |
| Zero to index mark instead of going to both limits | User must get axis within 2 seconds motion at high speed of the index mark. The system will move 2 seconds forward and 4 seconds in reverse looking for the index mark. If found it will zero the system and go to zero. If not it will go back to the original position with no change of the zero. Zeroing repeatability is less than 0.5 micron. |
| Add flipper mirror control to USB command set | Command /1M0R<CR> where the 0 is 0 or 1 will change the position of the flipper mirror. 0 is camera and 1 is laser. |
| Add flipper mirror query to USB command set | Command /1?mR<CR> will return /0%dR<CR> where %d is 0 or 1. 0 is camera and 1 is laser. |
| Add limit feedback to USB command set | When performing a /n?8R<CR> query (where n is the axis number) if a limit is active instead of a position the return will be /0LLR<CR> where LL stands for lower limit and UL for upper limit. |
| Add zeroing command to Windows utility | This allows the user to zero the rotation axis, as it is not allowed from the touch screen. X, Y, and Z can be zeroed this way as well. An offset can also be applied to any axis to move the zero away from the index mark. (i.e. for rotation if the index mark isn’t exactly level.) |